DIAMOND

# DESCRIPTION

# Monster 3D is the first generation of 3D graphics accelerator for all 3D games based on Direct3D™ and Glide™.

# With its add-on design Monster 3D can be added to any standard graphics adapter and connected with the supplied high-quality loop-trough cable.

# Based on the Voodoo graphics chipset (1Pixelfx, 1Texelfx) from 3Dfx, Monster 3D offers best 3D performance for current 3D action games supporting Direct3D™ and Glide™. The board contains 4 MB EDO DRAM.

Hardware

- 3Dfx Interactive Voodoo chip set

- PCI bus 2.x compliant

- 4 MB EDO DRAM on board for:

- 2 MB frame buffer & z-buffer

- 2 MB texture memory

- 135 MHz DAC

- Analog pass through (connection to standard VGA output)

###### MONSTER 3D

##### High Performance 3D Entertainment Accelerator

- PCI Add On Board

- Compatible with most Graphics Adapters

- Works with High Quality Analog Pass Through cable

- Supports MS Direct3D™ and Glide™ API

- High-performance 3D Full Screen Games Support

Monster 3D

Product Specification

Hardware Features

- Perspective correct texture mapping

- Bi-linear and advanced texture filtering

- Level of detail (LOD) MIP mapping

- Sub-pixel correction

- Polygonal-based Gouraud shading and texture modulation

- 24-bit internal, 16-bit dithered RGB frame buffer

- Multiple format texture buffer (8-bit and 16-bit)

- 1, 2, and 4 Mbytes of frame/aux buffer memory per pixelfx

- 1, 2, and 4 Mbytes of texture memory per texelfx

- 45 Mpixels/sec sustained fill rate for bilinear or advanced filtered textures

- 1,000,000 Textured triangles/sec on a P-166 PCI system

- Anti-aliasing

- Z-buffering (16-bit)

- Alpha blending

- Per-pixel special effects: fog, transparency, translucency

- Texture compositing, morphing, animation

- Linear frame buffer access

17. June 1998

http://www.diamondmm.de

DIAMOND

What you Get

- Monster 3D Graphics Add On Board

- High Quality Pass-Through cable

- CD-ROM with installation software and Online Manual

- Hardware installation manual

Retail Product without games and 
Retail Product with games available

- Psygnosis: Formula 1 full version

- Interplay: Descent ™ II - Destination Quartzon ™ (OEM version)

- Activision®

- Mech Warrior2® (full game)

- Hyperblade ™ (full game)

- Demo / Light version EF2000 ™

- Microsoft Game Sampler 2 with the trial versions of:

- Monster Truck Madness ™

- Hellbender ™

- and 14 other games ( demo or light versions)

- System Requirements

- PCI 2.x compatible motherboard

- Microsoft Windows® 95 or higher

- VGA graphics card

Display Modes

| Resolution | Colors | Refresh Rate (Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 | 72 | 75 | 85 | 120 |
| 512*384 | 65k |  |  |  |  |  |
| 640*400 | 65k |  |  |  |  |  |
| 640*480 | 65k |  |  |  |  |  |
| 800*600 | 65k |  |  |  |  | - |

Notes:

- Colors are 24 bit dithered to a 16 bit RGB (65k colors) output.

- All resolutions and refresh rates are valid for Windows® 95. Other operating systems may differ.

Monster 3D

©  1998, Diamond Multimedia Systems, Inc. Subject to technical alteration without notice

™ The brands or product names mentioned are trademarks or registered trademarks of their respective owners

Product Specification

Software

- Easy to use multilingual install program

- Driver for Microsoft Direct3D™

- Glide™ register level compatible driver software for full screen coin-op/arcade games within Windows® 95

Game Compatibility

- All Monster 3D games, using Glide

- All DirectX 5 games, using Direct3D will be accelerated by Monster 3D

Certifications

- FCC Class B DOC, CE, UL

Service & Support

- 5 Years Warranty

- Software Updates via Internet

- Fax on Demand System

- Technical Support-Hotline

## Germany

Diamond Multimedia Systems

Spea Software GmbH

Moosstraße 18 b

D  82319 Starnberg

Fax: +49 8151 21258

## UK

Diamond Multimedia Systems Ltd.

610 Wharfedale Road

Winnersh Triangle

Wokingham, Berkshire, RG41 5TY

Fax: +44 1189 444401

## France

Sarl Diamond Multimedia Systems France

122, Avenue du Général Leclerc

92100 Boulogne

Fax: +33 (0) 1 55 38 16 01

http://www.diamondmm.de