Stealth II S220

Product Specification (6. 11. 1997)

State of the art 3D graphics performance for less.

- Realistic 3D graphic in real-time for price conscious Windows® user

- Best of class 3D performance for your favourite 3D software

- Outrageous 2D Windows® acceleration

- Advanced Video Playback

- Ready for Diamond's DVD and DTV extensions

- Hardware

- High Performance 3D Graphics Accelerator

- Rendition V2100, 64-bit RISC graphics processor

- 170 MHz RAMDAC

- 4 MByte SGRAM (100 MHz)

- 33/66MHz PCI bus

- API: Microsoft Direct3D, Rendition Redline™ and Speedy3D™

- Horizontal Sync Signals 31,5kHz - 91kHz

- Vertical Sync Signals 43,5Hz - 120Hz

- VESA Power Management Support

- HighColour with 1280x1024 at 85Hz refresh rate

- TrueColour with 1024x768 at 85Hz refresh rate

- 2D Feature Set

- BitBlt, Line Draw, Polygon Fill

- Hardware cursor

- 3D Feature Set

- Flat and Gouraud Shading

- Texture Mapping with

- perspective Correction,

- Bilinear and Trilinear Filtering

- Mip mapping

- Transparency (colour key and alpha)

- Z-Buffering (16 bpz)

- Fogging

- Hardware triangle set-up

- Alpha Blending

- Anti-aliasing

- Video

- Scaling of digital with X and Y interpolation

- Hardware colour space conversion

- Video connector (VMI-A) for Diamond’s DVD decoder and digital TV tuner

- Software

- BIOS

- 32 KByte EPROM

- VESA Bios extension (VBE) v2.0 support

- DDC 2B plug & play

- Graphics Driver for

- Windows® 95 / 98, Turbo Driver

- Windows NT™  4.0

- Software for Windows® 95

- InControl Tools ( 5 Languages)

- DirectX 5 incl. Direct3D, DirectDraw and DirectVideo support

- Software MPEG-1 decoder

- VPE driver for DVD support

- Microsoft Internet Explorer

- Software for Windows NTÔ 4.0

- DirectX 3

- Microsoft Internet Explorer

- Driver for DOS

- MS-DOS mode centring and refresh tool

- MS-DOS diagnostic software

- Software Bundles

- Game Sampler CD with RRedline™ and Direct3D games

- Documentation

- HTML manual (5 languages)

- InControl Tools on-line help (5 languages)

- System Requirements

- Pentium™ P5-90 or better or other 100% compatible system

- 4 MB System RAM

- Windows® 95 or Windows NT™ 4.0

- One available PCI 2.1 extension slot

- No other active VGA or SVGA compatible graphics accelerator in the PC

- Compatibility

- 100% compatible with VGA, SVGA and VESA Standard

- Service & Support

- 5 years warranty

- Free Software updates via BBS, CompuServe and Internet:: //http/www.diamondmm.de

- Fax-on-Demand Service

- Technical support hot-line

- Available

- October 1997

Display Modes

| Resolution | Colours | 60 Hz | 70 Hz | 72 Hz | 75 Hz | 85 Hz | 120 Hz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480 | 256 | 4 |  | 4 | 4 | 4 | 4 |
|  | 64k | 4 |  | 4 | 4 | 4 | 4 |
|  | 16M | 4 |  | 4 | 4 | 4 | 4 |
| 800x600 | 256 | 4 |  | 4 | 4 | 4 |  |
|  | 64k | 4 |  | 4 | 4 | 4 |  |
|  | 16M | 4 |  | 4 | 4 | 4 |  |
| 1024x768 | 256 | 4 | 4 |  | 4 | 4 |  |
|  | 64k | 4 | 4 |  | 4 | 4 |  |
|  | 16M | 4 | 4 |  | 4 | 4 |  |
| 1152x864 | 256 | 4 | 4 |  | 4 | 4 |  |
|  | 64k | 4 | 4 |  | 4 | 4 |  |
|  | 16M | 4 | 4 |  |  |  |  |
| 1280x1024 | 256 | 4 |  | 4 | 4 | 4 |  |
|  | 64k | 4 |  | 4 | 4 | 4 |  |
| 1600x1200 | 256 | 4 |  |  |  |  |  |
|  | 64k | 4 |  |  |  |  |  |

Windows®  95 modes; not necessary available for other operating systems.

Germany:

SPEA Software GmbH

a Diamond Multimedia Company

Moosstraße 18 b

82319 Starnberg - GERMANY

Fax: ++49 8151 21258

UK:

Diamond Multimedia Systems Ltd.

610 Wharfedale Road

Winnersh Triangle

Wokingham

Berkshire RG41 5TY  UK

Fax: ++44 1189 444401

France:

Diamond Multimedia Southern Europe

6, Blvd. Général Leclerc - Bat C - 15iéme Etage

Clichy 92115 - FRANCE

Fax: ++33-1-47561139

Copyright

© 1997 Diamond Multimedia Systems, Inc.  Subject to technical alterations

Trademarks

(ä) The brands or product names mentioned are/may be trademarks or registered trademarks of their respective holders.