ENGINEERING RELEASE NOTES

S3 ProSavage

(4.13.01.3007-13.00.07) 10/10/2001

Recipient

S3

Purpose

Engineering release

Products Supported

- S3 ProSavage PM133

- S3 ProSavage KM133

Resolutions Supported

Mode list for 100Mhz Memory Clock

*320x200	8,16,32	70

*320x240	8,16,32	72

*400x300	8,16,32	72

*512x384	8,16,32	70

*640x400	8,16,32	70

*720x480	8,16,32	75

*720x576	8,16,32	75

640x480	8	60, 72, 75, 85, 100, 160

800x600	8	56, 60, 72, 75, 85, 100, 160

848x480	8	75

1024x768	8	60, 70, 72, 75, 85, 100, 130

1152x864	8	60, 70, 72, 75, 85, 100

1280x1024	8	60, 72, 75, 85, 100

1600x1200	8	60, 75

640x480	16	60, 72, 75, 85, 100, 160

800x600	16	56, 60, 72, 75, 85, 100

848x480	16	75

1024x768	16	60, 70, 72, 75, 85

1152x864	16	60, 70, 72, 75, 85,

1280x1024	16	60, 72, 75, 85 (72,75,85 with no overlay support in 100Mhz)

640x480	32	60, 72, 75, 85, 100, 160

800x600	32	56, 60, 72, 75, 85, 100

848x480	32	75

1024x768	32	60, 70, 72, 75 (75 with no overlay support in 100Mhz)

Mode list for 133Mhz Memory Clock, In addition to 100 Mhz mode list

1024x768 	16 	100

1024x768	32	85

1152x864	16	100

1280x1024	16	100

1600x1200	16	60, 75, 85 (75, 85 with no overlay support in 133Mhz)

* The modes are DirectDraw modes.

* The modes are DirectDraw modes.

Software Requirements

Win95/Win98/WinMe

DirectX 7.0 required

Earlier versions of DirectX are supported, but are not tested or recommended for use with this driver version.

Language Support

- Full Language Support

ENU, CHS, CHT, DAN, DEU, ESP, FIN, FRA, ITA, JPN, KOR, NLD, NOR, PTB, SVE, THA

Package Definition for S3 Standard Package with Utilities

Files included in package without utilities is indicated by **

DATA.TAG

DATA1.CAB

DVIDEO.DLL**

LANG.DAT

LAYOUT.BIN

NBICD.DL_**

OS.DAT

S3GAMCHS.HLP

S3GAMCHT.HLP

S3GAMDAN.HLP

S3GAMDEU.HLP

S3GAMENU.HLP

S3GAMESP.HLP

S3GAMFIN.HLP

S3GAMFRA.HLP

S3GAMITA.HLP

S3GAMJPN.HLP

S3GAMKOR.HLP

S3GAMMA.CFG

S3GAMMA.DLL

S3GAMNLD.HLP

S3GAMNOR.HLP

S3GAMPTB.HLP

S3GAMSVE.HLP

S3GAMTHA.HLP

S3HAV.DLL**

S3SETUP.DAT

S3SW2CHS.HLP

S3SW2CHT.HLP

S3SW2DAN.HLP

S3SW2DEU.HLP

S3SW2ENU.HLP

S3SW2ESP.HLP

S3SW2EXT.CFG

S3SW2FIN.HLP

S3SW2FRA.HLP

S3SW2ITA.HLP

S3SW2JPN.HLP

S3SW2KOR.HLP

S3SW2NLD.HLP

S3SW2NOR.HLP

S3SW2PTB.HLP

S3SW2SVE.HLP

S3SW2THA.HLP

S3SWTCH2.DLL

S3SWTCH2.CFG

S3SWTCH2.VXD

S3UNINST.EXE

S3VDD8NB.VXD**

S3VID.DLL**

S3_8NB.DLL**

S3_8NB.DRV**

SAVAGENB.INF**

SETUP.INI

SETUP.INS

SETUP.LID

SETUP.EXE

SETUPDIR

VIAGART.VXD**

_INST16.EX_

_ISDEL.EXE

_SYS1.CAB

_USER1.CAB

Additions and Changes

Version: 4.13.01.3007-13.00.07

Date: 10/10/2001

PERFORCE Label: Win9x_4.13.01.3007-13.00.07

-fix for P4M DVD Player Bug

-Fix Stream processor problem under TV-Out 256C mode.

-work around bug on P4 or twister:sysmark randomly hang up by resending  command instead of automatically repeat.

-Disabled DXVA support in Win9x

-Add a Cursor_move var to block cursor movement during hibernate

-add a patch not to set aspect ratio=1 in order to run space haste

-fix the bug that can't change TV type to PAL mode with BT869.

-Open the 1152x864, 1280x768, 1280x960, 1360x768 mode for P4M.

Version: 4.13.01.3006-13.00.06

Date: 10/04/2001

PERFORCE Label: Win9x_4.13.01.3006-13.00.06

CR43188 	DCT400a  test : DDRAW: OVERFLY failed.

Version: 4.13.01.3005-13.00.05

Date: 9/27/2001

PERFORCE Label: Win9x_4.13.01.3005-13.00.05

The screen will flash when play *.avi/*.mpeg file and open the MS-DOS prompt at once.

Add fix for "We press hot-key to switchdisplay in 640x480  while playing DVD. We will see wrong overlay"

To fix the bug:"When CRT+TV, set resolution 1600*1200 and then switch to 1024*768, the CRT&TV no display and system hang."Because Twister only have one display.

Dolphin GPF from monitor off

Version: 4.13.01.3004-13.00.04

Date: 9/20/2001

PERFORCE Label: Win9x_4.13.01.3004-13.00.04

Updated P4M BCI PM value to 3f01.

Version: 4.13.01.3003-13.00.03

Date: 9/14/2001

PERFORCE Label: Win9x_4.13.01.3003-13.00.03

PM133\KM133 Win98\WinME 10.31.17 driver set: General: Signability failed.

Windows can start with only 16 colors mode.

A most right vertical line is not displayed when system makes an overlay surface.

A middle of screen is corrupted when mediaplayer window is moved to out of screen on the digital  LCD with SCALER.

Installation Instructions

Use standard Win9x driver installation procedures for driver installations.

VIA VGART.VXD is installed when the driver is installed.

Developer(s)

S3 Win9x team.

©Copyright 2001 S3 Graphics, Inc. All rights reserved. If you have received this document from S3 Graphics Inc. in electronic form, you are permitted to make the following copies for business use related to products of S3 Graphics: one copy onto your computer for the purpose of on-line viewing, and one printed copy. With respect to all documents, whether received in hard copy or electronic form, other use, copying or storage, in whole or in part, by any means electronic, mechanical, photocopying or otherwise, is not permitted without the prior written consent of S3 Graphics Inc., 2481 Mission College Boulevard., Santa Clara CA 95054. S3 is a registered trademark of S3 Incorporated. DuoView, Streams Processor , Savage, Savage4, ProSavage, SuperSavage, and Twister are trademarks of S3 Graphics Inc. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Graphics reserves the right to make changes in the product design without reservation and without notice to its users.