Notes on REFRESH.EXE, VMODE.EXE and TLIVESA.COM

=======================================================================

REFRESH                  Tseng Labs ET4000 family Refresh Rate Setup Program

Version 1.5                    (C)Copyright 1994 BRITEK, All Rights Reserved

Usage: REFRESH				(For manual setup)

REFRESH <resolution> <v_freq.>	(For command line setup)

REFRESH R -or- REFRESH r		(Show current-setting)

REFRESH/?			(Show this help screen)

�����������������������������������������������������������������������

<resolution>			<v_freq.>

640:  640 x  480		0, 1, 2, 3, 4

800:  800 x  600		(See Note as follow)

1024: 1024 x  768

1280: 1280 x 1024

�����������������������������������������������������������������������

Note:	 640 x  480: 0=60 Hz, 1=72 Hz, 2=75 Hz, 3=90 Hz

800 x  600: 0=56 Hz, 1=60 Hz, 2=72 Hz, 3=75 Hz, 4=90 Hz

1024 x  768: 0=87 Hz (Interlaced), 1=60 Hz, 2=70 Hz, 3=75 Hz

1280 x 1024: 0=87 Hz (Interlaced), 1=60 Hz, 2=70 Hz, 3=75 Hz

�����������������������������������������������������������������������

Example: REFRESH 1024 3

(For 1024x768, 75Hz Non-interlaced mode)

===========================================================================

USING VMODE TO CHANGE DISPLAY MODES AND SCAN RATES

The Tseng VGA provides you with the capability to utilize extended column

modes with text applications. This means that your Tseng VGA board, when

interfaced with appropriate color displays, can produce 132x44,

132x28, 132x25, 100x40, and 80x60 modes in addition to the standard

80x25 and 40x25 modes. With the appropriate software, the 132-column

display capability allows emulation of terminals that require 132

columns of text.

With the Tseng VGA board, you can switch back and forth between the 80-

column display modes and the various extended-column display modes. To

use VMODE, follow these steps:

1. First, be sure that the VMODE.COM utility is present on the disk

you are using.

2. To install VMODE.COM for the first time type VMODE INSTALL

and then press ENTER.

VMODE will display the following:

Display Type:

1. VGA display

2. Variable-frequency display

Which type of display are you using?

Press F1 after you have made your selection, or press

<Esc> to quit.

3. To switch to a different mode, type the mode you wish to use and

then press ENTER:

Type

VMODE 25      - to switch to 132x25 mode

VMODE 28      - to switch to 132x28 mode

VMODE 44      - to switch to 132x44 mode

VMODE 40      - to switch to 40x25 mode

VMODE 60      - to switch to 80x60 mode*

VMODE 80      - to switch to 80x25 mode

VMODE 100     - to switch to 100x40 mode*

* Note: Not all modes are supported by all monitors. Attempting to use

modes which your monitor does not support will produce unsatisfactory

results.

Three additional modes are provided to enable you to change the scan

line resolution. This is to accommodate some software that

specifically looks for a particular number of scan lines. The 400 scan

line mode provides the most pleasing text resolution.

VMODE 200 - to provide 200 scan lines in 40- or 80x25 text modes.

VMODE 350 - to provide 350 scan lines in 40- or 80x25 text modes.

VMODE 400 - to provide 400 scan lines in 40- or 80x25 text modes.

SELECTING DISPLAY SCAN RATES

Vmode includes commands to provide additional scan rates that

may improve synchronization with a variety of monitors. Normally, the

default scan rates are effective, but some monitors may require

different scan rates for the most satisfactory display results. By

setting these modes, frequencies are adjusted that affect displayed

modes. The following are available via the VMODE command.

You may wish to add to your autoexec.bat file the Vmode command and parameter

for the best display possible on your monitor.

example: VMODE x

VMODE     Modes         Vertical        Horizontal

x Value   affected      Refresh Rate    Frequency   Resolution

35K       29,2A,30      56Hz            35KHz       800x600 *

38K       29,2A,30      60Hz            38KHz       800x600 *

48K       29,2A,30      72.7Hz          48KHz       800x600 *

45M       37,38         87Hz            35.5KHz     1024x768 interlaced *

65M       37,38         60.5Hz          49.0KHz     1024x768 non-interlaced *

72M       37,38         69.8Hz          56.3KHz     1024x768 non-interlaced *

72H       11,12,25,2E   72.7Hz          38.7KHz     640x480 *

1280-I	  3F		87Hz		48.1KHz	    1280x1024 interlaced *

1280-60   3F		60Hz		64.4KHz	    1280x1024 *

1280-70   3F		70Hz		75KHz	    1280x1024 *

* Note: Not all modes are supported by all monitors. Attempting to use

modes which your monitor does not support will produce unsatisfactory

results.

Typing VMODE ? will display the full array of mode possibilities.

Users that are familiar with the characteristics of their monitors

will find these modes more useful.

NOTE:  Choose modes/resolutions that are compatible with your video

board configuration and monitor capabilities.

USING THE COMPATIBILITY MODES

Some programs are written to be run using specific modes or are

written according to particular video standards. For example, there

are programs that require a monochrome adapter or those that are

written exclusively for a Color Graphics Adapter. When these kinds of

programs are used, it becomes necessary to make your VGA board

appear to be what the program requires. This is easily accomplished.

In order to select a mode that will change the "appearance" of the

video adapter to the software, you need only select the appropriate

VMODE parameter. When another mode is needed or desired, you can

simply select the mode using another VMODE parameter. In order to

return to the default mode (VGA), type VMODE VGA or power down (the

VGA mode will be in effect upon power-up). The following is a list of

modes and their uses. A complete listing of available modes can be

seen by typing VMODE ?.

VMODE CGA -     To set the adapter for compatibility with the Color

Graphics Adapter.

VMODE MDA -     To set the adapter for compatibility with the

Monochrome Display Adapter.

VMODE HERCULES -To set the adapter for compatibility with the Hercules

card.

VMODE EGA -     To set the adapter for compatibility with the Enhanced

Graphics Adapter.

VMODE VGA -     To set the adapter for compatibility with the Video

Graphics Array.

A typical use might be for using game software that requires CGA

resolution. Using VMODE CGA will put the VGA board in CGA mode. If

the software needs to be booted in order to work, you simply place the

VGA in CGA-compatible mode with VMODE CGA, and warm-boot with your

game diskette in the A: drive.

You may switch modes in this manner as often as you wish. VMODE ? will

display a list of the available modes. Entering VMODE with no

parameter will display a menu screen from which to choose a text mode.

==========================================================================

VESA Super VGA BIOS Extension Driver

TLIVESA.COM is the VESA Super VGA BIOS Extension driver for video

boards based on Tseng Labs ET3000 and ET4000 chips. The driver

implements Version 1.2 of the VESA specification and is a

terminate-and-stay-resident (TSR) program which intercepts the video

BIOS interrupt vector to provide additional BIOS commands supporting

super-VGA modes.

To load, type the command:

TLIVESA [ENTER]

at the DOS prompt. TLIVESA may be loaded through a batch file for

convenience. After loading the driver, any VESA application can be

run.

The program can also be unloaded, freeing up memory, provided no other

resident program which intercepts the video BIOS interrupt vector is

loaded afterward. (Actually, no resident program at all should be

loaded afterward in order to really free the memory.)

To unload, type the command:

TLIVESA U [ENTER]

Unloading TLIVESA may also be performed through a batch file.