To install the LynnSoft SmartEnabler from file to UnixWare:

-----------------------------------------------------------

1. Go to the /tmp in UnixWare system.

cd /tmp				<Enter>

2. Copy the unwlsen.tar file from dos diskette to /tmp directory:

doscp a:unwlsen.tar unwlsen.tar	<Enter>

3. Detar the file:

tar xvf unwlsen.tar		<Enter>

4. Run the pkgadd command as following:

pkgadd -d /tmp/unwlsen		<Enter>

5. Follow the standard pkgadd installation procedure.

Note: For now lets install from file after everything is OK

I will make you a pkgadd format diskette.

To build the SCO custom diskette for LynnSoft SmartEnabler do the following:

---------------------------------------------------------------------------

1. Go to the /tmp in SCO UNIX system.

cd /tmp				<Enter>

2. Copy the scolsen.tar file from dos diskette to /tmp directory:

doscp a:scolsen.tar scolsen.tar	<Enter>

3. Detar the file:

tar xvf scolsen.tar		<Enter>

4. Go to scolsen directory:

cd /tmp/scolsen			<Enter>

5. Now build the custom diskette by inserting the formated

diskette into the driver and type:

Driver A (or drive 0)

---------------------

For 1.2M diskette:

tar c2v .		<Enter>

For 1.44M diskette:

tar c6v .		<Enter>

For 720K diskette:

tar c4v .		<Enter>

Driver B (or drive 1)

---------------------

For 1.2M diskette:

tar c3v .		<Enter>

For 1.44M diskette:

tar c7v .		<Enter>

For 720K diskette:

tar c5v .		<Enter>

5. Install the driver using custom command

Driver A (or drive 0)

---------------------

For 1.2M diskette:

custom -m /dev/rfd096ds15	<Enter>

For 1.44M diskette:

custom -m /dev/rfd0135ds18	<Enter>

For 720K diskette:

custom -m /dev/rfd096ds9	<Enter>

Driver B (or drive 1)

---------------------

For 1.2M diskette:

custom -m /dev/rfd196ds15	<Enter>

For 1.44M diskette:

custom -m /dev/rfd1135ds18	<Enter>

For 720K diskette:

custom -m /dev/rfd196ds9	<Enter>

Note: 	This driver for SCO it will auto-detect the installed

NE2000 or (nat0) driver and set the IO and IRQ

accordingly.  If the NE2000 or nat0 is not install yet

it will prompt user and set the IO and IRQ to default

at IO=0x300 and IRQ=5.

Note also:  Please remove the OLD driver before install the new driver.