******************************************************************************

*              Advanced NE2000 Compatible Card -- Packet Driver              *

******************************************************************************

Use  the  packet driver with Advanced NE2000 Compatible Card.

To load the packet driver use the following syntax:

GE3PD <packet_int_no>

where:

<packet_int_no>    This specifies the software interrupt (IRQ)

where  you  will load the  packet  driver.  It

must be in the range 0x60 to 0x80.

The  PC/TCP kernel, by default, is  loaded  at

interrupt  0x61  (so  don't  load  the  packet

driver there!)

Example:

The following command line loads the packet driver at software

interrupt 60h:

GE3PD 0x60

Note: The placement of 0x prior to the parameters.

After the packet driver is loaded, you can rung the PC/TCP kernel

ETHDRV.EXE.  The PC is then ready for PC/TCP communications.